Product Specifications

RHODIUM SERIES

Indoor P4 • 16:9 aspect ratio LED Display

Overview

Vanguard’s Rhodium Series has a 16:9 native aspect ratio. If cabinets are arrayed with equal cabinet height & width (i.e. 10x10 or 8x8), the entire display will retain a 16:9 viewing aspect ratio. Rhodium is available in pixel pitches of P2.5 and P4.

Rhodium can be serviced from either the front or rear of the cabinet. Cabinets can utilize external jumpers from cabinet to cabinet. Alternatively, the cabinets may be completely internally cabled in order to achieve a flat back for wall installations. Rhodium cabinets can be wall mounted, flown or ground stacked. Quick Locks provide easy installation. Pre-threaded mounting location make installation simple. Magnetic LED modules make module replacement quick & easy.

Additional Features

- Fan-less design.

- Low Power Consumption.

- Designed for maximum heat dissipation.

- Pixel to Pixel color calibration for brightness and color uniformity.

- Seam brightness compensation is standard.

- High gray scale & high refresh rate.

- Flicker fee for broadcast applications.

- Black LED’s for high contrast ratios and sharp image quality.

- Low profile mask offers very wide viewing angles.

- 16-bit color processing technology.

- Superb color & brightness uniformity.

Rhodium Technical Specifications

Rhodium Indoor P4

CABINET

Cabinet Material	             Die Cast Aluminum

Cabinet Size	      	                        25.2” x 14.7”

Cabinet Resolution   	                                160 x 90

Modules Per Cabinet			                4

Cabinet Weight				     19.1lbs.

Service Access			         Front or Rear

IP Rating				           IP31

EXTERNAL CONNECTIONS

Data in / out		   		      RJ45

AC Power in / out		            PowerCon

ENVIRONMENTAL

Operating Temperature		      -10°C to +50°C

Storage Temperature		      -20°C to +60°C

Operating Humidity		     20% to 90% RH

AC POWER

Source Voltage		              100-240V, 50/60Hz

Max Power Consumption		         200W

Avg. Power Consumption	   	         100W

INTERNAL OPERATION

Driving Method			     1/15th Scanning

Gray Scale				14 – 18 bit

IC Driver		                              MB15153

Refresh Rate				   >3840Hz

Frame Frequency			          60Hz

LED

LED Type		                    SMD2121 Black

LED Configuration	                                 3 in one

Pixel Density		                            62,600/m²

Module Size		                        12.6” x 7.35”

Module Resolution	                                    80x45

Modules Per Cabinet	                                             4

Lifetime to Half Brightness (L50)	                  100,000

Contrast Ratio				    2,000:1

Brightness			 	     800Nits

Viewing Angle			             140° x 140°

Minimum Viewing Distance		      13.2 ft.

Rhodium Technical Specifications