Product Specifications

AXION II ChipFlip SERIES

Indoor P0.9 Fine Pixel Pitch LED Display

• 16:9 aspect ratio

Axion II ChipFlip Features

- “ChipFlip” is a revolutionary new technology of encapsulating fine pixel pitch SMD 1010 LED lamps to PCB boards.

- Traditionally the circuit design for SMD displays for both the Positive pole, the Anode, and the Negative pole, the Cathode, face upwards.
However, the circuit design for “ChipFlip” for both the Positive and Negative electrodes face downward and the corresponding encapsulation is called the “ChipFlip” Process.

- The strength of the solder used in the “ChipFlip” Process” is 100 times stronger that that normally used in the SMD process.

- Vanguard will guarantee a failure rate of 5/PPM in year 1, 10/PPM in year 2 and 15/PPM in year 3. The failure rate for SMD displays is 50-100 PPM in year 1.

- The “ChipFlip Process” also employs Common Cathode technology whereby the electric current passes through the Cathode, the negative pole or the ground.

- Power is not on continouously but only draws power as needed. As a result, heat generation is greatly reduced.

- The contrast ratio and color gamut of " Chipflip ", featuring true HDR technology are comparable to the Samsung Wall and Sony’s Crystal LED.

- Brightness levels can reach 4000 nits. Typically only outdoor displays can reach those levels.

Axion Technical Specifications

Axion II ChipFlip P0.9

Specifications

INTERNAL CONNECTIONS

Axion has no external connections

100% internal connections inside cabinet

Data in / out		   		      RJ45

AC Power in / out		                       IEC

CABINET

Aspect Ratio				          16:9

Cabinet Material	             Die Cast Aluminum

Cabinet Size	      	      23.62” x 13.28” x 2.28”

Cabinet Resolution   	                              640 x 360

Cabinet Weight				       17 lbs.

Service Access			         	         Front

IP Rating				           IP30

ENVIRONMENTAL

Operating Temperature		      -10°C to +50°C

Storage Temperature		      -20°C to +60°C

Operating Humidity		     20% to 90% RH

AC POWER

Source Voltage		              100-240V, 50/60Hz

Max Power Consumption		         165W

Avg. Power Consumption	   	           85W

INTERNAL OPERATION

Driving Method			     1/32th Scanning

Gray Scale				   14-18 bit

IC Driver				 MBI5153

Refresh Rate				   >3840Hz

Frame Frequency			          60Hz

LED

LED Type		                    	 SMD1010

LED Configuration	                                 3 in one

Pixel Density		                        1,131,734/m²

Module Size		                       11.81” x 6.64”

Module Resolution	                              320 x 180

Modules Per Cabinet	                                             4

Lifetime to Half Brightness (L50)	                   100,000

Contrast Ratio			                   10,000:1

Brightness			           600-2500 Nits

Viewing Angle			               160° x 160°

Minimum Viewing Distance	         	         3.2 ft.